# AXAF / EPHIN

# EPHIN PHA BUFFER DESCRIPTION

# Institut für Experimentelle und Angewandte Physik

# University of Kiel

# March 23, 1999

Document Change Log

### Revision		Date			Change Description		Prepared by

## Initial Release	March 23, 1999	Initial Release			C. Scharmberg

TABLE OF CONTENTS

1. EPHIN SCIENTIFIC DATA BLOCK	4

2. EPHIN PHA BYTE TYPES	4

2.1. T: Coincidence and LSB’s of A and B type	5

2.2. D: Direction and A, B status type	5

2.3. A: A detector MSB type	5

2.4. B: B detector MSB type	5

2.5. C: C detector MSB type	6

2.6. D: D detector MSB type	6

2.7. E: E detector MSB type	6

2.8. X: C, D status and LSB’s type	6

2.9 Y: E status and LSB type	6

3. EPHIN PHA DATA SETS	6

4. DETECTOR ENERGY RANGE	7

5. PRIORITY BUFFER	7

6. PHA BUFFER EXAMPLE	7

- EPHIN SCIENTIFIC DATA BLOCK

The EPHIN scientific data block has a length of 1290 bytes. These bytes contains digital housekeeping data, a priority pointer, single detectors and coincidence countrates, histogram data , and the Pulse Height Analysis (PHA) buffer.
The PHA buffer is divided into two parts. The first part contains the statistical sample, the second part contains priority coincidences. At the beginning of each accumulation period the PHA buffer will be filled chronological with the statistical sample coincidences, until there are only eight or less bytes of free memory. Then the onboard software will fill the same memory starting with the highest address in reverse order with some priority coincidences. Obviously some of the statistical samples are overwritten with priority coincidences. The priority pointer bytes contains the Priority Pointer Address  of the last written priority data set. If there is no priority coincidence in the memory block, the priority pointer contains ‘1151’.

| Byte position | Priority Pointer Address | Contents |
| --- | --- | --- |
| 1-5 |  | Digital housekeeping |
| 6 |  | Low byte Priority Pointer |
| 7 |  | High byte Priority Pointer |
| 8-91 |  | Countrates |
| 92-139 |  | Histogram data |
| 140 | 0 | First byte of PHA buffer |
| ... | ... | ... |
| ... | ... | ... |
| 1290 | 1150 | Last byte of PHA buffer |

- EPHIN PHA BYTE TYPES

Depending on the coincidence conditions one PHA data set contains between 4 and 9 bytes. Each bytes is one of the following 9 types:

T: Coincidence and LSB’s of A and B type

D: Direction and A, B status type

A: A detector MSB type

B: B detector MSB type

C: C detector MSB type

D: D detector MSB type

E: E detector MSB type

X: C,D status and LSB’s  type

Y: E status and LSB type

A9 ... A0, B9 ... B0, C9 ... C0, D9 ... D0, E9 ... E0: 10 bit pulse height values

2.1.	T: Coincidence and LSB’s of A and B type

MSB

| T3 | T2 | T1 | T0 | A1 | A0 | B1 | B0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

T3	T2	T1	T0	Coincidence

0	0	0	0	E150

0	0	0	1	E300

0	0	1	0	E1300

0	0	1	1	E3000

0	1	0	0	P4

0	1	0	1	P8

0	1	1	0	P25

0	1	1	1	P41

1	0	0	0	H4

1	0	0	1	H8

1	0	1	0	H25

1	0	1	1	H41

1	1	0	0	INT

2.2.	D: Direction and A, B status type

MSB

| DA2 | DA1 | DA0 | DB2 | DB1 | DB0 | SA | SB |
| --- | --- | --- | --- | --- | --- | --- | --- |

DA2	DA1	DA0	Detector Segment

0	0	0	A00

0	0	1	A01

.	.	.

1	0	1	A05

DB2	DB1	DB0	Detector Segment

0	0	0	B00

0	0	1	B01

.	.	.

1	0	1	B05

SA	Status Channel A

- low energy range

- high energy range

SB	Status Channel B

0	low energy range

1	high energy range

2.3.	A: A detector MSB type

MSB

| A9 | A8 | A7 | A6 | A5 | A4 | A3 | A2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

MSB’s detector A

2.4.	B: B detector MSB type

MSB

| B9 | B8 | B7 | B6 | B5 | B4 | B3 | B2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

MSB’s detector B

2.5.	C: C detector MSB type

MSB

| C9 | C8 | C7 | C6 | C5 | C4 | C3 | C2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

MSB’s detector C

2.6.	D: D detector MSB type

MSB

| D9 | D8 | D7 | D6 | D5 | D4 | D3 | D2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

MSB’s detector D

2.7.	E: E detector MSB type

MSB

| E9 | E8 | E7 | E6 | E5 | E4 | E3 | E2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

MSB’s detector E

2.8.	X: C, D status and LSB’s type

MSB

| C1 | C0 | D1 | D0 | SC | SD | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- |

SC	Status Channel C

- low energy range

- high energy range

SD	Status Channel D

0	low energy range

1	high energy range

2.9.	Y: E status and LSB type

MSB

| E1 | E0 | SE | X | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- |

SE	Status Channel E

0	low energy range

1	high energy range

- EPHIN PHA DATA SETS

The number of bytes for one data set depends on the coincidence condition and varies between 4 and 9. In all cases the first byte includes the coincidence information, and therefore the number of following bytes belonging to the same data set is defined.

| Coincidence | PHA byte types | Number of bytes |
| --- | --- | --- |
| E150, P4, H4 | T D A B | 4 |
| E300, P8, H8 | T D X A B C | 6 |
| E1300, P25, H25 | T D X A B C D | 7 |
| E3000, P41, H41 | T D X Y A B C D E | 9 |
| INT | T D X Y A B C D E | 9 |

- DETECTOR ENERGY RANGE

The energy lost in one detector can be calculated by multiplying the 10 bit pulse height value times the maximal energy divided by 1023. Attention: The maximal  energy depends on the status bit (low range or high range).

| Maximal energy [MeV] | A detector | B detector | C detector | D detector | E detector |
| --- | --- | --- | --- | --- | --- |
| ##### Low range | 3 | 3 | 16.07 | 20 | 20 |
| High range | 30 | 45 | 166.7 | 225 | 225 |

- PRIORITY BUFFER

The priority buffer may contain a maximum of the following data sets:

#### Coincidence		Maximum number of data sets

E150				4

E300				4

E1300				4

E3000				4

P4				4

P8				4

P25				4

P41				4

H4				40

H8				16

H25				4

H41				4

INT 				0

- PHA BUFFER EXAMPLE

These example buffer contains only 2 priority coincidences. The priority pointer contains 1143

| Address | Hex | Type | Description |  |
| --- | --- | --- | --- | --- |
| 0 | 2F | T | Coincidence = E1300  length = 7 bytes, A0=A1=B1=B0=1 | 1st statistical sample |
| 1 | AC | D | Segments A5 and B3, A and B in low energy range |  |
| 2 | F0 | X | C0=C1=D1=D0=1, C and D in low energy range |  |
| 3 | 12 | A | 0001001000 = 72  energgy lost = 72 * 3 / 1023 = 0.211 MeV |  |
| 4 | 27 | B | 0010011100 = 159  energy lost = 159 * 3 / 1023 = 0.466 MeV |  |
| 5 | A1 | C | 1010000100 = 644  energy lost = 10.116 MeV |  |
| 6 | 9E | D | 1001111000 = 632  energy lost = 12.356 MeV |  |
| 7 | 0F | T | Coincidence = E150  length = 4 bytes, A0=A1=B1=B0=1 | 2nd statistical sample |
| 8 | 29 | D | Segments A1 and B2, A in low-, B in high- energy range |  |
| 9 | AB | A | 1010110111 = 695  energgy lost = 695 * 3 / 1023 = 2.038 MeV |  |
| 10 | 77 | B | 0111011111 = 487  energy lost = 487 * 45 / 1023 = 21.422 MeV |  |
| ... |  |  |  |  |
| ... |  |  |  |  |
| ... |  |  |  |  |
| 1143 | 90 | T | Coincidence = H8  length = 4 bytes, A0=A1=B1=B0=0 | 2nd prio. coincidence |
| 1144 | 4B | D | Segments A2 and B2, A and B in high energy range |  |
| 1145 | 70 | A | 0111000000 = 224  energgy lost = 224 * 30 / 1023 = 6.569 MeV |  |
| 1146 | A0 | B | 1010000000 = 640  energy lost = 640 * 45 / 1023 = 28.152 MeV |  |
| 1147 | 8F | T | Coincidence = H4  length = 4 bytes, A0=A1=B1=B0=1 | 1st prio. coincidence |
| 1148 | AC | D | Segments A5 and B3, A and B in low energy range |  |
| 1149 | 10 | A | 0001000011 = 67  energgy lost = 67 * 3 / 1023 = 0.196 MeV |  |
| 1150 | 55 | B | 0101010111 = 343  energy lost = 343 * 3 / 1023 = 1.006 MeV |  |